60G Radar Module

MS60-1211S80M-Parking

Product manual

Document Revision History

| Version | Revision Content | Revision time |
| --- | --- | --- |
| V1.0 | Pre-release version | 2025.9.2 |

Product Catalogue

1 Product Overview	1

1.1 Product Introduction	1

1.2 Function Description	1

2 Technical Specifications	1

2.1 Radar Parameters	2

2.2 Electrical Characteristics	2

3 Module dimensions and pin descriptions	2

3.1 Size and Packaging	2

3.2 Pin Description	4

3.3 Output parameters	4

4 Typical Applications	5

4.1 Basic Functions	5

4.2 Application Expansion	5

4.3 Typical application circuit	6

4.4 Customization options	6

5 Installation Method	6

5.1 Horizontal installation	6

5.2 Inclined Installation	9

6 Important Notes	9

- Product Overview

  - Product Introduction

MS60-1211S80M-Parking is a high-performance 60GHz millimeter-wave radar sensor module launched by Daicheng Technology. It is based on the 1T2R frequency-modulated continuous wave signal processing mechanism. By measuring the centimeter-level distance of targets within the detection range and perception, it can achieve the function of reversing distance measurement radar.

Figure 1-MS60-1211S80M-Parking

  - Function Description

Reverse vehicle target positioning:

This radar module can output real-time position information of the closest target within the detection range, including distance and angle.

- Technical Specifications

  - Radar Parameters

| Parameters | Numerical value | Unit |
| --- | --- | --- |
| Working frequency | 59~64 | GHz |
| Bandwidth | 5（Adjustable） | GHz |
| Modulated waveform | FMCW | / |

| Transmission power | 11 | dBm |
| --- | --- | --- |
| Horizontal | ±60 | ° |
| Vertical | ±60 | ° |
| Detection Range | ≤3 | m |

  - Electrical Characteristics

| Parameters | Numerical value | Unit |
| --- | --- | --- |
| Working voltage | 3.0~5.5 | V |
| Average working current | 80（Adjustable） | mA |
| Operating temperature | -30~85 | ℃ |
| Storage temperature | -40~125 | ℃ |
| Communication interface | GPIO， UART | / |

- Module dimensions and pin descriptions

  - Size and Packaging

Figure 2 -Schematic diagram of radar module structure

    - Volume (L*W*H, mm): 12.0 * 11.0 * 2.0

    - Interface: Dual row 1*3 straight insertion / stamp hole interface

  - Pin Description

Figure 3- Pin Definitions for Radar Module

| Interface | Pin | Name | Type | Explanation |
| --- | --- | --- | --- | --- |
| Interface 1 | 1 | OUT | GPIO | Signal output |
|  | 2 | GND | GND | Power ground |
|  | 3 | VCC | PWR | Power supply |
| Interface 2 | 1 | TX | UART | Serial port output |
|  | 2 | GND | GND | Power ground |
|  | 3 | RX | UART | Serial port input |

  - Output parameters

    - Target information: Distance, Angle

    - Product Information

- Typical Applications

  - Basic Functions

Support target positioning

The multi-channel millimeter-wave radar has a relatively good angle measurement capability. This radar module uses two antennas to receive signals and measure the phase difference of the signals' arrival times. Through phase comparison, angle estimation (DOA) is achieved. This technology can accurately measure the horizontal angle of the target object relative to the radar. Combined with the high-precision ranging capability, it enables the positioning of the target in a two-dimensional space.

  - Application Expansion

This radar module can be installed independently at the rear of the vehicle for use in reversing scenarios. It can provide real-time position information of the nearest obstacle detected by the radar, including distance and angle. It can also be linked with the reversing camera to offer the owner stable detection of reversing scenarios.

Figure 4 -Example of the application of this radar module in the reversing scenario

  - Typical application circuit

Figure 5- Schematic Diagram of the Connection of Peripheral Devices for This Radar Module

  - Customization options

| Power supply voltage | Inductive output mode | Adjustment method | Cheating function |
| --- | --- | --- | --- |
| ☑3.3V | ☑ IO Output | □ IO Adjust parameters | □Photosensitive |
| ☑ 5V | ☑ UART | ☑ UART | □ Power management |
| □12V | □ PWM | □ IIC | —— |
| □ 24V | —— | —— | —— |
| Note：□ Supportable ☑ Supported |  |  |  |

- Installation Method

This radar module is only compatible with horizontal installation and inclined installation.

  - Horizontal installation

When installed horizontally, the radar module is attached to the rear of the vehicle, with its viewing angle facing horizontally towards the monitoring area. This is suitable for vehicles with a relatively low chassis such as compact cars.

In this installation mode, the maximum detection distance for targets is 3 meters, and the near-range blind area is 0.15 meters.

Figure 6 -Schematic diagram of the flat packaging of this radar module

Suggested installation location：

- Installation height：0.5~1.0m

- Detection range：≤3m

- Detection blind zone：0.15m

  - Inclined Installation

When installed at an angle, the radar module needs to be given a certain inclination angle. This is mainly applicable for monitoring from a higher position downward or from a distance. It is generally suitable for vehicles with a relatively high chassis, such as trucks, to conduct detection and monitoring from top to bottom within the space. The detection area will be affected by installation factors such as height and inclination angle, resulting in different area ranges.

In this installation mode, the maximum detection distance of the target is 2.5m, and the near blind zone is 0.15m.

Suggested installation location:

    - Installation height：0.8~1.5m

    - Inclination angle：15°~30°

    - Detection range：≤2.5m

    - Detection blind zone：0.15m

Figure 7 -Schematic diagram of the inclined installation of this radar module

- Important Notes

- During the installation and operation of the radar module, the inner and outer surfaces of the secondary cover should be smooth and flat, with uniform thickness. For flat or spherical surfaces, they should not be uneven; if there is a surface coating, it should not contain any metal or conductive materials.

- Within the radar FOV coverage area, there needs to be a good clear space in the antenna radiation area to avoid any metal. The material casing or support components should be used to prevent interference.

- Try to avoid pointing the radar module antenna directly at large metal equipment or pipes, etc., to avoid false alarms.

- Due to the radiation characteristics of the radar antenna, the radar detection range is distributed in a water-drop shape starting from the radar antenna surface. Deviating from the normal direction of the radar center line, the effective range will be reduced.

- The physical system of millimeter waves affects the radar module's operating range, which is related to the target's reflection intensity, the material and thickness of the target's covering, etc. The effective range of the radar will fluctuate to some extent. For the detection of living targets, different body positions and states will affect the radar's operating range. The radar does not guarantee that it can reach the maximum operating range in all states.

- When multiple radar modules are installed, it is best to avoid mutual irradiation between the antennas of each module and keep a distance of more than 1m between the modules.